UAS Safety & Risk Assessment

SORA 2.5 Assignment

Mission Ditlevsdal Bison Farm wants to use an eBee X for BVLOS flights to monitor bison. Complete a high-level SORA assessment by working through all 10 steps. Goal: Aim for the largest useful Flight Geography you can justify while keeping the operation and resulting SAIL proportionate. Keep answers short and explain your reasoning.

Step 1: Proposed Operation / Concept of Operations (ConOps)

What is the operation trying to achieve?

Where will you operate? Define the intended Flight Geography.

What is the maximum operating height?

Which UAS will you use?

Tip: Define the volumes before doing the risk calculations. The Operational Volume is the Flight Geography + Contingency Volume. The Ground Risk Class footprint is the Operational Volume + Ground Risk Buffer.

SORA semantic model map

Show the Flight Geography, Contingency Volume, Ground Risk Buffer and Adjacent Area. Add the main height limit as a note.

Sketch or paste your map here

Step 2: Intrinsic Ground Risk Class (iGRC)

What is the highest population density within the iGRC footprint?

What is the maximum speed of the UAS?

What is the maximum characteristic dimension of the UAS?

What is the resulting iGRC?

Tip: Use SORA Table 2. For a fixed-wing UAS, the characteristic dimension is the wingspan. Use the designer-defined maximum possible commanded airspeed, not just your planned cruise speed. If the footprint crosses areas with different population densities, use the highest relevant density.

Step 3: Final Ground Risk Class (GRC)

Which ground-risk mitigations are genuinely applicable? Add a short justification or supporting evidence.

| Mitigation | Applicable? | Why / supporting evidence? |
| --- | --- | --- |
| M1(A) Strategic mitigation - sheltering |  |  |
| M1(B) Strategic mitigation - operational restrictions |  |  |
| M1(C) Tactical mitigation - ground observation |  |  |
| M2 Reduced effects of UA impact dynamics |  |  |

For this exercise, you can state your assumed robustness rather than working through Annex B in detail. Apply any claimed credits in the sequence given in SORA Table 5.

What is the final GRC after the applicable mitigations?

Step 4: Initial Air Risk Class (iARC)

Which initial ARC applies to the Operational Volume, and why?

Are there any relevant NOTAMs, temporary restrictions or special airspace constraints?

Tip: Use dronezoner.dk as your starting point and follow the SORA Figure 6 logic. Check the maximum height, controlled/uncontrolled airspace, airport/heliport environment, urban/rural environment and whether the airspace is atypical. Assess the full Operational Volume, not only the nominal route.

Step 5: Strategic Mitigations and Residual ARC

Which strategic mitigation(s) could reduce the air encounter rate?

What is the residual ARC after those mitigations?

Tip: Think about changes made before flight, such as time of operation, geographic or altitude restrictions, or airspace structure/coordination. Only reduce the ARC if you can justify that the local encounter rate is lower than the initial classification.

Step 6: Tactical Mitigation Performance Requirement (TMPR)

What TMPR level and robustness follow from your residual ARC?

| ARC-a | ARC-b | ARC-c | ARC-d |
| --- | --- | --- | --- |
| No requirement | Low | Medium | High |

In one or two sentences, what would you use to detect and respond to manned traffic during the BVLOS flight?

Tip: Keep this at concept level. Describe the detection/information source and the avoidance response. SORA discusses options such as DAA, cooperative traffic information and operational procedures; the required performance depends on the residual ARC.

Step 7: Specific Assurance and Integrity Level (SAIL)

Final GRC:

Residual ARC:

Resulting SAIL:

Tip: Use SORA Table 7: the SAIL is derived from the final GRC and residual ARC. Do not choose a SAIL first and work backwards.

Step 8: Containment Requirements

What is the lateral size of the Adjacent Area?

What population / assembly-of-people limits are relevant in the Adjacent Area?

What containment robustness is required?

Tip: For a UA above 250 g, SORA 2.5 sets the Adjacent Area outer limit using the distance flown in 3 minutes at maximum speed, with a minimum of 5 km and maximum of 35 km. Also check the average population density in the Adjacent Area and outdoor assemblies within 1 km of the Operational Volume. The 1 km distance is an assembly-of-people check, not the size of the Adjacent Area.

Step 9: Operational Safety Objectives (OSOs)

Use SORA Table 14 to enter the required level (NR / L / M / H). For each OSO that is required, give one short example of how you would implement it or what evidence you would keep. A few words are enough.

| OSO | Short description | Level | Implementation / evidence example |
| --- | --- | --- | --- |
| #01 | Operator competence / proven organisation |  |  |
| #02 | UAS manufactured by competent/proven entity |  |  |
| #03 | UAS maintained by competent/proven entity |  |  |
| #04 | Safety-essential components designed to an ADS |  |  |
| #05 | System safety and reliability considered in the design |  |  |
| #06 | C3 link characteristics appropriate for the operation |  |  |
| #07 | Conformity check of the UAS configuration |  |  |
| #08 | Operational procedures defined, validated and followed |  |  |
| #09 | Remote crew trained and current |  |  |
| #13 | External services adequate for the operation |  |  |
| #16 | Multi-crew coordination |  |  |
| #17 | Remote crew fit to operate |  |  |
| #18 | Automatic flight-envelope protection from human error |  |  |
| #19 | Safe recovery from human error |  |  |
| #20 | Human factors / HMI evaluation |  |  |
| #23 | Environmental limits defined, measurable and followed |  |  |
| #24 | UAS qualified for adverse environmental conditions |  |  |

Step 10: Comprehensive Safety Portfolio (CSP)

In 4-6 bullets, what would you include in the final CSP?

Tip: Think of the CSP as the final safety case: operation description -> safety claims and mitigations -> derived requirements -> compliance evidence -> clear traceability between them. You do not need to build the full portfolio for this exercise.